Wig Stand Steps

John Griffiths

Coulee Region Woodturners

5/20/2023

- Need three pieces, top and bottom should end up being 5 ½” to 6 ½”  by 1 ½” or more thick.   Spindle can be ~8” to 12” long.  Great way to use scrap pieces of wood.  Bottom can be larger diameter and a bit heavier for stability.  2X6 pine construction lumber will work fine.  I would suggest that only the very brave use Douglas Fir.

- Design Considerations:  Make it fairly domed rather then flattish to better support the wig shape.

- Spindle: Can again be nearly anything.  Lengths of spindle can vary and I am someone who likes to work with tree branches so I salvaged various cutoffs from what falls off my trees in my yard.

- At this point have in mind the tenon and mortise size for assembly, as well as your choices for mounting the top and bottom.

- I drill a 1 1/8” mortise or hole in each blank about ¾” deep or a scoch deeper.  On the top I sometimes put on a glue block so as to preserve maximum thickness of the top for a most lifelike shape, opposite the 1 1/8” hole.  Reason for the hole being 1 1/8” is to fit my pin chuck.

- (Note: a few steps can be saved by drilling a 5/16” hole for a screw chuck, then turning the upper portion and leaving a tenon.  If you use this method you drill a hole or mortise of your choice for the spindle.  My preference is to drill the 1 1/8” hole for the pin chuck.)

- The top is turned but cutting a ~2 ¼” tenon at the top, reverse chucked and the underside of the top is trued, shaped and sanded to 320 grit.

- The top is reverse chucked in the pin jaws and the upper side is shaped, the tenon turned away and sanded to 320.

- Next we do the base.  I use the 1 1/8” Forstner for the hole in the top, or you can vary with the smaller hole for the screw chuck.  Mount and turn round. True the tailstock and headstock sides and then cut a slight concave in the bottom, then about a 2 ¼” mortise, decide on the shape of the side of the base and make it.  Leave some weight in the base so the stand will resist tipping. Sand to 320.

- Reverse chuck in expansion mode.  True the top and cut a “tray” or groove into the top to store jewelry.  Turn a nice transition from the hole to the tray. Sand to 320.

- Be certain to leave sufficient depth in the holes for the tenons and whatever glue oozes to the bottom of the hole and still allows a tight glue joint.

- The Spindle:  Women who have short hair wigs will not require a tall stand so a stand can have a shaft of perhaps 8” between tenons to about a foot for longer hair length.  Tenons I make about a half inch or a bit more. Remember to leave the tenon short enough so so excess glue at the bottom of the hole has someplace to go, allowing a nice close glue joint.

- I use a 1 1/8” diameter tenon again to match my Pin Jaw size.

- Your spindles can mix or match the top and base species.  I like to use a different species just because I can.  Again, I like to salvage limbs.  Use thick enough so you can cover the area around the holes, this adds grace to the piece also in my opinion, and we want the recipient to appreciate the overall shape and form.

- Mount your blank between centers, round off and turn the tenons.  Measure them to have a nice fit.  I have a rather fat Steb drive center so I leave a bit of wood between the head and tail stocks and the tenon which I whack off with the chop saw at the end.  Find a shape which can be nearly any spindle design you choose and when complete, sand the shaped part to 320. Sanding the tenons is a waste of time and may mess up your fit.

- The Denis Falch final turning step:  He chucks the base by the bottom tenon and places a vee cone live center in the tailstock.  He glues the stand together and compresses the vee tailstock against the top with a bit of foam to protect the wood.  When the glue is dry he turns a nice cove into the junction of the glue joints, top and bottom, then sands to 320.

- Finish considerations.  I am partial to polyurethne.  We want a finish that will withstand freshly washed wigs being hung on them without a chemical reaction between the finish and the cleaning solution.  I advise avoiding shellac because alcohol is it’s solvent and it may be some cleaning solutions contain alcohol.  Use a couple coats, the 320 grit surface will probably give a fine finished coat without much sanding.